Initial setup

General description of our configuration for general public.

Actually, such a lab environment could be configured in 5-10 minutes.

Zywall 310

LAN: 10.8.2.0/24

WAN: ….

Tunnel1 (GRE to WAN of Zywall 110):10.8.131.1/29

IPSec tunnel (IPSec between WAN interfaces of GWs) – initially inactivated

Phase 1 - PSK, AES256, SHA512, DH5

Phase 2 – Nailed up, Site-to-site, Enable GRE over IPSec, ESP, Tunnel, DES, SHA1, DH2

BGP: AS number – 64500, router ID - 10.8.131.1 , Redestribute – Off, neighbors - 10.8.131.2/ 64501, enable EBGP Multihop, Announcing networks - 192.168.0.0/16 & 10.0.0.0/8

Zywall 110

LAN: 10.8.3.0/24

WAN: ….

Tunnel1: (GRE to WAN of Zywall 310):10.8.131.2/29

IPSec tunnel (IPSec between WAN interfaces of GWs) – initially inactivated:

Phase 1 - PSK, AES256, SHA512, DH5

Phase 2 – Nailed up, Site-to-site, Enable GRE over IPSec, ESP, Tunnel, DES, SHA1, DH2

BGP: AS number – 64501, router ID - 10.8.131.2 , Redestribute – Off, neighbors - 10.8.131.1/ 64501, enable EBGP Multihop, Announcing networks - 10.8.3.0/24

Experiment 1: Un-encrypted GRE with BGP support

- Implement configurations: 310_Experimental1.conf  & 110_Experimental1.conf  respectively.

(Sorry, I cannot share my conf files to general public, because I got them from a productive environment)

- SSH to '310 gw

- Ping from 310 to 110 works fine:

Router310# ping 10.8.131.2

PING 10.8.131.2 (10.8.131.2) 56(84) bytes of data.

64 bytes from 10.8.131.2: icmp_seq=1 ttl=64 time=5.06 ms

64 bytes from 10.8.131.2: icmp_seq=2 ttl=64 time=4.87 ms

64 bytes from 10.8.131.2: icmp_seq=3 ttl=64 time=4.64 ms

- Check for BGP connectivity and announcing on 310:

Router310# show bgp summary

Neighbor        AS          MsgRcvd  MsgSent  TblVer   InQ    OutQ   Up/Down   State/PfxRcd

===============================================================================

10.8.131.2      64501       8        9        0        0      0      00:04:49  Established

Router310# show ip route bgp

Flags: A - Activated route, S - Static route, C - directly Connected

O - OSPF derived, R - RIP derived, B - BGP derived, G - selected Gateway

! - reject, H - Black hole, L - Loop

IP Address/Netmask    Gateway          IFace           Metric    Flags    Persist

===============================================================================

10.8.3.0/24           10.8.131.2       tunnel1         0         ABG      0000h:07m:48s

- Connect  to the opposite gw through the tunnel interface:

Router310# ssh admin@10.8.131.2

The authenticity of host '10.8.131.2 (10.8.131.2)' can't be established.

RSA key fingerprint is b2:e3:58:d7:49:4a:d6:e2:7b:f9:7b:f6:a1:8e:5a:fc.

Are you sure you want to continue connecting (yes/no)? yes

Warning: Permanently added '10.8.131.2' (RSA) to the list of known hosts.

Password:

- Check for BGP connectivity and announcing on 110:

Router110> enable

Router110# show bgp summary

Neighbor        AS          MsgRcvd  MsgSent  TblVer   InQ    OutQ   Up/Down   State/PfxRcd

===============================================================================

10.8.131.1      64500       14       16       0        0      0      00:11:04  Established

Router110# show ip route bgp

Flags: A - Activated route, S - Static route, C - directly Connected

O - OSPF derived, R - RIP derived, B - BGP derived, G - selected Gateway

! - reject, H - Black hole, L - Loop

IP Address/Netmask    Gateway          IFace           Metric    Flags    Persist

===============================================================================

10.0.0.0/8            10.8.131.1       tunnel1         0         ABG      0000h:11m:19s

192.168.0.0/16        10.8.131.1       tunnel1         0         ABG      0000h:11m:19s

- Check for routing between some hosts on LANs of 310 and 110:

C:\Windows\system32>ipconfig

Windows IP Configuration

Ethernet adapter Ethernet:

Connection-specific DNS Suffix  . :

IPv4 Address. . . . . . . . . . . : 10.8.2.10

Subnet Mask . . . . . . . . . . . : 255.255.255.0

Default Gateway . . . . . . . . . : 10.8.2.254

C:\Windows\system32>ping 10.8.3.115

Pinging 10.8.3.115 with 32 bytes of data:

Reply from 10.8.3.115: bytes=32 time=4ms TTL=126

Reply from 10.8.3.115: bytes=32 time=4ms TTL=126

Reply from 10.8.3.115: bytes=32 time=12ms TTL=126

Reply from 10.8.3.115: bytes=32 time=17ms TTL=126

Ping statistics for 10.8.3.115:

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss),

Approximate round trip times in milli-seconds:

Minimum = 4ms, Maximum = 17ms, Average = 9ms

So, un-encrypted GRE with  BGP support works fine!!!

Experiment 2: GRE over IPSEC. BGP does not work.

- Implement configurations: 310_Experimental2.conf  & 110_Experimental2.conf  respectively.

Or

Just activate  IPSec VPN rules "Domodedovo_GRE_over_IPSec" and "Greenwood_GRE_over_IPSec" on 310 and 110 respectively and put down and up tunnel interfaces on both routers to re-establish GRE tunnel through IPSec.

- Ping from 310 to 110 works fine:

Router310# ping 10.8.131.2

PING 10.8.131.2 (10.8.131.2) 56(84) bytes of data.

64 bytes from 10.8.131.2: icmp_seq=1 ttl=64 time=5.06 ms

64 bytes from 10.8.131.2: icmp_seq=2 ttl=64 time=4.87 ms

64 bytes from 10.8.131.2: icmp_seq=3 ttl=64 time=4.64 ms

- Check for BGP connectivity and announcing on 310:

Router310# show bgp summary

Neighbor        AS          MsgRcvd  MsgSent  TblVer   InQ    OutQ   Up/Down   State/PfxRcd

===============================================================================

10.8.131.2      64501       35       39       0        0      0      00:03:25  Connect

Router310# show ip route bgp

Flags: A - Activated route, S - Static route, C - directly Connected

O - OSPF derived, R - RIP derived, B - BGP derived, G - selected Gateway

! - reject, H - Black hole, L - Loop

IP Address/Netmask    Gateway          IFace           Metric    Flags    Persist

===============================================================================

- Try to SSH from 310 to 110:

Router310# ssh admin@10.8.131.2

ssh: connect to host 10.8.131.2 port 22: Connection timed out

- Check for BGP connectivity and announcing on 110:

Router310# show bgp summary

Neighbor        AS          MsgRcvd  MsgSent  TblVer   InQ    OutQ   Up/Down   State/PfxRcd

===============================================================================

10.8.131.1      64500       34       39       0        0      0      00:01:17  Connect

Router# show ip route bgp

Flags: A - Activated route, S - Static route, C - directly Connected

O - OSPF derived, R - RIP derived, B - BGP derived, G - selected Gateway

! - reject, H - Black hole, L - Loop

IP Address/Netmask    Gateway          IFace           Metric    Flags    Persist

===============================================================================

AT THIS POINT WE HAVE STATED A PROBLEM WITH NO SOLUTION

Additional experiments:

- Packet capture:

- Route the traffic using Policy route:

Activate policy route #1 on BOTH 310' and 110' GWs.

Check for routing status:

And check for routing between some hosts on LANs of 310 and 110:

C:\Windows\system32>ipconfig

Windows IP Configuration

Ethernet adapter Ethernet:

Connection-specific DNS Suffix  . :

IPv4 Address. . . . . . . . . . . : 10.8.2.10

Subnet Mask . . . . . . . . . . . : 255.255.255.0

Default Gateway . . . . . . . . . : 10.8.2.254

C:\Windows\system32>ping 10.8.3.115

Pinging 10.8.3.115 with 32 bytes of data:

Reply from 10.8.3.115: bytes=32 time=4ms TTL=126

Reply from 10.8.3.115: bytes=32 time=4ms TTL=126

Reply from 10.8.3.115: bytes=32 time=12ms TTL=126

Reply from 10.8.3.115: bytes=32 time=17ms TTL=126

Ping statistics for 10.8.3.115:

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss),

Approximate round trip times in milli-seconds:

Minimum = 4ms, Maximum = 17ms, Average = 9ms